Darren’s Winter Check List

(Hobie 16)

- Check Side stays, trapeze wires and mainsail halyard for broken strands (If so replace)

- Check your split rings on your chain plate etc. have not been twisted out of shape. (If so then replace)

- Check chain plates for cracks and rust

- Make sure side stay anchor bolts and bow bolts are not loose. (They should be firm & not over tight)

- Check trapeze ropes and elastics for chafing. (replace if necessary)

- Tighten dolphin striker (Very firm)

- Tighten trampoline ropes as much as possible

- Make sure rudder bolts are firm ( so no side movement but can still be raised and lowered easily)

- Check rudder pins are not worn (if so either replace or turn upside down)

- Check the bungs to see if the rubber seal has perished (If so then replace)

- Check your rudder cams for cracks and if no cracks then spray with silicone spray or grease with Teflon grease. (If cracked or broken then replace)

- Check your rear beam rivets that they are not loose and are still there. (If so then replace)

- Check to make sure all cleats and pulleys are in good working order, free from sand and corrosion

- Give your boat a good clean. “CIF” is great, and then give it a quick polish to protect it from the sun this summer. (A Teflon polish is great)

- Fix up any major dings in your hulls (self explanatory)